How to Write a new Code Plug to your Anytone Radio

- Download the latest Anytone Code Plug from the WA7DEM website.

- Go to https://support.bridgecomsystems.com/ and download the latest CPS (Customer Programming Software) and Firmware package for your radio.

- Unzip (extract) the software you just downloaded.

- Open the folder that contains the software you just extracted and then open the folder that contains the CPS.

- Click on Setup.exe and follow the prompts to install the CPS software.

- Launch the CPS Software.

- In the CPS, click the folder icon. Then locate and open the Code Plug you downloaded earlier.

- Enter your DMR ID and Call Sign

  - In the CPS menu, click on DIGITAL and then click on RADIO ID LIST.

  - Click on 1 and an editor will open

  - Enter Your DMR ID in the ID field and your Call Sign in the NAME Field and then click OK

- Connect your Computer to your radio and turn the radio on.

- In the CPS, click on SET then SET COM and select the COM PORT assigned to your radio.

- Click on PROGRAM then click on WRITE TO RADIO.

- Select OTHER DATA and click OK.

- The CPS will now write the code plug to the radio and inform you when it is complete.